PRINCIPLE OF OPERATION

The circuit comprises of three sections.

- light controlled circuit

- counter  for  timer

- switching circuit

Light controlled circuit:

This particular circuit controls other circuits. it puts the other circuit on or off depending the  nature of the day. It comprises of ldr and transistor representation of smith trigger configured using two npn transistor(bc377) in such that when there is a small potential difference across  the ldr and the variable resistor, the relay will be triggered onto switch on the other circuit. The circuit monitors day and night. The main function of this monitor is put off the device at night and on in the mornining.

counter  for  timer:

Since sun start from the east at 6:45am and set at exertly 12:45pm, the light controller circuit put on the whole circuit automatically making the timer to initialize its 6hrs counting sequence.

switching circuit:

Microcontroller through p3.6  of at89c52 ,in accordance to the program in its memory, activate p3.2 of  AT89C2051 that is in charge of eastward, then the controller(at89c2051)in turn  in turn activate R1 and R4, whereby turning the motor eastward to expose the solar plate to full access on the sun and stop by the eastward limiter.imidiately it is 12:45pm,microcontroller timer drop to 0:00 (that is 6hrs gap)and activate western p3.3 of at89c2051 through p3.5 of at89c52 .the same sequence repeat  and turn the motor westward and stop by the west limiter in other to expose the plate to full exposure to the sun. When the motor is turning eastward,R1 and R4 are on at the same time

R4 is sensor relay that make sure that   R3 and R2 are off to avoid R2 oning at the same time with R1.

R3 plays the same roll to switch R1 and R24off when R2 is on

Base on the directional movement of the motor,+ve and-ve are connected to the motor through R4 dual pole and the connection is interchanged through R3 dual pole to westward.

Type of motor used:

24-36v d.c Actuator from super jack used in tv satalite dish polar adjustment