Introduction:

What is robot:

Robot is a mechanical device that sometimes resembles a human and is capable of performing a variety of often complex human tasks on command or by being programmed in advance.

Project is about:

Here at this robot I have used a Bluetooth module to control the robot via 2 BO motors at 100RPM appox the robot is control by an android phone application

Microcontroller used is AT89S52 form 8051 family to work in a serial communication UART mode the communication is configured on 9800bps to communicate it with the Bluetooth module

The Bluetooth module used is a HC-05 in smd package which works on a 3.3v and have a serial communication with any device connected to it the communication speed can be configured on various speed via AT Command.

The BT module is a SPP supported profile so it can be connected easily to any module or phone. In this profile the data can be sent and receive to module. The BT module is connected to the RX pin of microcontroller.

The L293D is a motor driver IC to operate the motors in any direction required dependent on the logic applied to the logic pins.

A readymade compact size chassis I have used to avoid the chassis assembly the chassis contains 2 decks the lower is used for BO motors fitting the upper is used as a battery stack .on top the plate the board is mounted by screw fitting.

Block Diagram:

BT module

Motor 2

Motor 1

L293D

Motor     driver

AT89S52

Micro

Circuit Diagram for Bluetooth module:

The reset ckt at pin 11 and KEY  ckt at pin24 can be ignored to work the module in communication mode.

Motor driver ckt:

The pin 8 of IC should b connected to the 9v battery or 12v

This pin8 is internally connected to the driver ckt inside the IC which helps the moter to get the good supply  which also helps the smooth functioning of motors

The complete circuit:

The crystal circuit and the reset circuit is as common usually used in every AT89S52  circuit

The leds connected to the moter indicates the flow of current….the direction of the moter  rotation.

The Bluetooth module:

The Bluetooth module used is a HC-05 based on SPP support

Features:

1) Wireless serial bluetooth port.

2) With free power adapter bottom board come with well power regulator.

User can connect 3.3 to 5VDC and connect TX and RX to your control IO (general 3.3 to 5V digital input output of MCU or  IO is ok, or general TLL IO)

3) Easy to connect this module with PC, just search and key "1234" passcode.

4) With white SMD LED on the adapter board, can see the Bluetooth connection status.

Step to connect:

1) Connect the wiring, power up, while the device is not connected, the bluetooth module board has a white LED flashing

2) At PC side, search bluetooth device.

3) Found name called "HC-05" device

4) Connect it, and passcode is "1234"

5) While connection is ok, you can see the LED become always on

Usage:

- Coupled Mode: Two modules will establish communication automatically when powered.

- PC hosted mode: Pair the module with bluetooth dongle directly as virtual serial.

- Bluetooth protocal :  Bluetooth Specification v2.0+EDR

- Frequency :  2.4GHz ISM band

- Modulation :  GFSK(Gaussian Frequency Shift Keying)

- Speed : Asynchronous:  2.1Mbps(Max) / 160 kbps, Synchronous: 1Mbps/1Mbps

- Security :  Authentication and encryption

- Profiles :  Bluetooth serial port

- CSR chip : Bluetooth v2.0

- Wave band : 2.4GHz-2.8GHz, ISM Band

- Protocol : Bluetooth V2.0

- Voltage : 5V (3.6V-6V, NO more than 7V)

- User defined Baud rate :  4800, 9600, 19200, 38400, 57600, 115200, 230400,460800,921600 ,1382400.

Pin definition :

- PIO8 connects with LED cathodea with 470ohm series resistor in between. LED NEGATIVE connects to ground. It is used to indicate the module state. After powered on, flashing intervals differ in different states.

- PIO9 is used to control LED indicating paring. It will be steady on when paring is successful.

- PIO11, module state switching pin. HIGH -> response to AT command; LOW or floating -> regular work status.

- With build-in reset circuit, reset is completed automatically after powered on.

- Steps to set to MASTER:

- Set PIO11 HIGH with a 10K resistor in between.

- Power on, module comes into AT Command Response Status

- Open HyperTerminal or other serial tool, set the baud rate 38400, 8 data bits, 1 stop bit, no parity bit, no Flow

AT COMMONDS to check the Module:

- Set PIO11 HIGH with a 10K resistor in between.

- Power on, module comes into AT Command Response Status

- Open HyperTerminal or other serial tool, set the baud rate 38400, 8 data bits, 1 stop bit, no parity bit, no Flow

- Now u r enter in AT commond mode.

Command    AT

Response    OK 	the module is connected successfully

Commond    AT+ROLE?

Response     +ROLE=0	the module is in slave mode

Commond  AT+ROLE=1

Response    OK		the module is set in master mode

For more commande refer data sheet ……;)

In slave mode the PC,Phone etc will search the module and pair with the module

In master mode the module will search for any PC,phone etc

Code used:

org 0000h		organize from 0 locatin in rom

mov p0,#09h		mov data 09 to port 0 to test the working of micro

mov p3,#00h

mov p2,#00h

mov TMOD,#20h		put micro in UART mode at 9800 speed

mov th1,#-3

mov scon,#50h

setb tr1

here:

jnb ri,here		see wether the any data is reciver or not if not stay in loop

mov a,sbuf		copy recived data to acc

clr ri

cjne a,#'U',one 		compare that the recive data is U char,if no the jump to “one” and compare with the other

mov p2,#89h		if yes then send data 89 to port2 which forwarder the bot

sjmp here

one:

cjne a,#'D',two

mov p2,#86h

sjmp here

two:

cjne a,#'R',three

mov p2,#8ah

sjmp here

three:

cjne a,#'L',four

mov p2,#85h

sjmp here

four:

cjne a,#'C',here

mov p2,#00h

sjmp here